# Testing on E90-DTU(900SL30) – Configuration Mode and Transmission Functions

Prepared by

PM Ir Dr Tee Kian Sek

Date: 24 February 2021

Table of Contents

1	Objectives	3

2	References	3

3	The setup	3

4	Operating Mode	4

5	Configuration Software	5

6	Interpretation of the settings	6

# Objectives

- To study LoRa unit and its configuration settings.

- To figure out transmission functions [fixed, broadcast (broadcast address | Monitor address)].

# References

- E90-DTU(900SL30)_UserManual_EN_v1.0.pdf

- E90-DTU(900SL30)_UserManual_CN_V1.1.pdf

- Configuration software for E90-DTU, RF_Setting(E22-E90(SL)) V2.0.exe

- Product website, https://www.ebyte.com/

# The Setup

For configuration, we need:

- A DC12V power supply (here- an adaptor is used)

- RS232-to-USB converter

- E90-DTU

- A laptop (Window 10)

Like figure below.

For trials on transmission functions [fixed, broadcast (broadcast address | Monitor address)], setup as figure below. The aim is to choose a transmission function for 1 master 3 clients (1:3 mode), where the client nodes read sensory data and send to the master node.

Note: Figure below shows four nodes. They are assembled on two wooden pallet neatly. Good for presentation.

# The Configuration

## Operating Modes

Information retrieved from the user manual.

Figure 1: Configuration Modes

Before power ON, switch the function operating selector switches to [OFF, ON]. Power ON, connect RS232-to-USB converter between E90-DTU and a laptop. Click open the configuration software provided by the company.

## Configuration Software

Make sure E90-DTU is switched to Mode 2 - Configuration Mode. The configuration software would not read/write any parameters if not in Mode 2.

- Click open the configuration software, RF_Setting(E22-E90(SL)) V2.0.exe.

- Plug in RS232-to-USB converter to any USB-port and notice the COM port number. Select the right port number.

- At the configuration software, select the COM port and click “Open”.

- Once connected, click “Get” to read all parameters from E90-DTU.

Figure 2: Configuration software by Ebyte

Before “Open”. Choose the right COM port.

Figure 3: Reading configuration settings

After “Get”. This is the default factory settings.

Note:

Default factory settings may be different with the description stated in the user manual.

## Interpretation of the Settings

Refer to the user manual – E90-DTU.

The data structure: (In byte)

| command | Start address | length | Parameter |
| --- | --- | --- | --- |

Note: parameter means the registers being read. Refer to [4.4 Definition of Registers].

From [4.2 Configuration Software], the default parameter: (in Hex)

0xc0 0x00 0x09 0x00 0x00 0x00 0x62 0x00 0x12 0x03 0x00 0x 00

The interpretation:

| 0xc0 | 0x00 | 0x09 | 0x00 | 0x00 | 0x00 |
| --- | --- | --- | --- | --- | --- |
| Command | Start Address | Length | ADDH | ADDL | NETID |

| 0x62 | 0x00 | 0x12 | 0x03 | 0x00 | 0x00 |
| --- | --- | --- | --- | --- | --- |
| REG0 | REG1 | REG2 | REG3 | CRYPT_H | CRYPT_L |

The data structure reads from address 0x00 and followed by data reading of length 9-bytes.

## Definition of Registers

Refer to User Manual.

## Using a Serial Terminal

Operating mode : Mode 2

Instead of using the provided configuration software, the configuration software, RF_Setting(E22-E90(SL)) V2.0.exe., it is possible to use a typical serial terminal to read/write E90-DTU.

Referring to the user manual for data structure and registers.

Figure 4: Configuring using a serial terminal

- Sending HEX. Example: C1 00 09

- Receive: C1 00 09 00 00 00 62 00 12 03 00 00

- Refer to E90-DTU structure and registers

# The Transmission Functions

To figure out transmission functions [fixed, broadcast (broadcast address | Monitor address)].

Materials: 4 units of E90-DTU(900SL30)

Objectives: To arrange a network of 1:3, where all clients send data to a master.

Client 1

Client 2

Client 3

Master

Figure 5: My LoRa network and naming

## Default Settings

Operation Mode: 0

Configuration of E90-DTU: (default) C1 00 09 00 00 00 62 00 12 03 00 00

By default:

- all E90-DTU are in “Transparent Transmission”.

- Address = 0x00 0x00

- Channel = 0x12

- Baud rate = 9600, 8N1

- And others see user manual.

Note: See “Fixed Transmission” for difference.

### Method

- Label E90-DTU as “Master”, “Client 1”, “Client 2, “Client 3”.

- Power ON all E90-DTU.

- Connect each E90-DTU to a laptop via RS232-to-USB or RS485-2-USB (The reason: I have two units of RS232-to-USB and two units of RS485-2-USB.)

- At laptop, open four windows of serial terminals.

### The results

Figure 6: Results deploying default settings

### Comments

- No configuration is made.

- Each node can send data; and being received by other nodes.

- No designation of master or client node.

- Each node could broad-cast the data.

- Received data have no meta-data about the sender. No identifier.

### Suggestion

- If the default settings are applied for 1:3 network (1 master to receive 3 clients), the data need to carry identifier.

Example:

Data structure: [identifier] [separator] [data]

“N1, xxxx,  yyyy, ….”

- Design of data structure

Node 1  “N1, xxxx, yyyy”

Node 2  “N2, xxxx, yyyy”

Node 3  “N3, xxxx, yyyy”

Node 4  “N4, xxxx, yyyy”

## Fixed Transmission

### Method

- Configuration

  - Switch each node to Mode 2 (Configuration Mode).

  - Set the address and channel as indicated in the figure below.

  - [address, channel]

Master [0001, 02], Client 1 [0003, 04], Client 2[0005, 04], Client 3[0007, 06]

  - Set to “Fix Transmission”  - see REG3

    - First 3 bytes would be taken as serial data as: address high + address low + channel + data

    - Ex: 00 03 04 AA BB CC

Figure 7: Fixed Transmission and configuration (from user manual)

- Switch to Mode 0 (Normal Mode).

  - Open multiple serial terminal and monitor each node

  - Send from master (in Hex)

    - Note: each node  is set as fixed transmission.

    - Try sending to target and data

    - Example  00 03 04 AA BB CC

  - Verify the data received at each node.

### Results

Configuration of fixed transmission – using configuration software, RF_Setting(E22-E90(SL)) V2.0.exe.

Figure below shows the configuration of Master node. [address, channel] = Master [0001, 02].

The parameters are: Refer to [4.4	Definition of Registers].

| 0xc0 | 0x00 | 0x09 | 0x00 | 0x01 | 0x00 |
| --- | --- | --- | --- | --- | --- |
| Command | Start Address | Length | ADDH | ADDL | NETID |

| 0x62 | 0x00 | 0x02 | 0x43 | 0x00 | 0x00 |
| --- | --- | --- | --- | --- | --- |
| REG0 | REG1 | REG2 | REG3 | CRYPT_H | CRYPT_L |

Figure 8: Configuration of Fixed Transmission

Figure 9: The results of Master and three other clients

### Comments

- Since all nodes has different address, we could specify a target and send data.

- Example –

  - Master to client 1

  - Client 1 to client 2

  - Many more, only in 1 to 1 direction.

- The received data has no identifier. No idea from which source.

  - Example

  - Master send data = AA BB CC

  - Client 1 received = AA BB CC

  - Receiver has no identifier about the source.

### Suggestion

- Sent data is suggested to carry identifier as proposed in [5.1.4	Suggestion]

## Broadcasting Transmission

### Method

The network and naming as [5	The Transmission Functions]

Setting as Address, Channel and Fixed Mode

- Switch to Mode 0 (Normal Mode) after configuration.

  - Open multiple serial terminal and monitor each node

  - Send from master (in Hex)

    - Note: each node is set as fixed transmission.

    - Try sending to target and data

    - Example  FF FF 04 AA BB CC

  - By indicating target address as FF FF and Channel = 04

    - All address with Channel = 4 would receive the data

### Results

### Comments

- By sending from master, FF FF 04 abcd   all clients with channel = 04, would receive the data.

- The data sent and received has no identifier about the source.

### Suggestion

- Sent data is suggested to carry identifier as proposed in [5.1.4	Suggestion]